DTMF Remote Control Decoder

This kit provides an interface between any audio path capable of handling Dual Tone Multiple Frequency (DTMF) coding and switched outputs using relays.

Features

A 4 digit PIN code enables control access to the unit, this PIN code can be changed remotely.

A timeout function on the PIN code which can also be remotely changed to between 5 seconds and 255 seconds (4:15 minutes).

A command sequence end function that overrides the timeout function to prevent “follow on”.

Individual control of up to 8 relays comprising Operate, Release,Timed Toggling and Clock operation.

A full 16 DTMF keypad is required for full access operation although operation with a normal telephone style 12 key unit can be arranged.

Commands

# - Using this key at any point aborts the current command and resets the unit requiring the PIN code to be re-entered.  Note: the # key cannot be used within a PIN code.

PIN code - After entering the required 4 digit PIN code the following commands are available;

Relay control – A digit between 1 and 8 selects a relay for the following digit entered to control,

the second digit can be;

0 – Release relay

1 – Operate relay

2 – Toggle use of the relay using the relay toggle times.

3 – Switch using Clock times

Any other value will cause the command to abort and the system to reset, requiring the PIN to be re-entered.

Change PIN – The “A” key followed by a further 4 keys (other than #), these 4 digits must then be repeated before the new PIN is saved, once saved the system resets requiring the new PIN to be entered before other commands can be issued.  The default PIN as supplied is “CCCC”.

Example:  CCCC A 14AD 14AD

If the new PIN is entered incorrectly the second time then the system will reset with no changes.

Change Command Timeout – The “B” key followed by the number of seconds required, between 5 and 255, ending with the “*” key.  The new timeout period will come into operation after entering the PIN next time.

Change Relay Toggle times – The “C” key followed by the number of seconds the relay(s) are to operate for, followed by “*”, followed by the number of seconds the relay(s) are to be released for followed by “*”.

When using Relay Toggle, adding a relay to the group of relays to be switched, will cause a new cycle to begin immediately.  A relay can be removed from the group of relays being switched, by either using the Relay Toggle a second time in which case the relay will stay in its current setting, or by using the Release command or Operate command which will force the relay to the requested position.

Change Clock times – The “D” key followed by the current time in 24:00 format, followed by the On time in 24:00 format and finally the Off time in 24:00 format. I.e.   D 1339 0500 1900 would set the current time to 1:39 PM the On time at 5:00 AM and the Off time to 7:00 PM.

When using the Clock times use of the Operate, Release or Toggle functions is permitted at any time without altering the Clock mode.  To exit clock mode for a relay then the n3 command is used a second time.  Any number of relays can be set to operate at the specified times.

Contact Geoff Mather G8DHE  for further details  DTMFRemote@g8dhe.cix.co.uk                 Copyright 2002